TEST EVIDENCE DOCUMENT

AWS RDS PostgreSQL Multi-AZ Failover — Test Evidence

Ignition Platform Integration (Sanofi IIoT)

| Reference | Jira IIOTCLDEDG-1222 |
| --- | --- |
| Environment | DEV (dev-ignition-infrastructure) |
| Prepared by | Rohit Dwivedi |
| Date | 2026-07-25 |

# 1. Purpose

This document provides test evidence that the AWS RDS PostgreSQL database backing the Ignition platform is deployed in a Multi-AZ redundant configuration, and that a forced failover of the primary instance completes successfully with continued application-level connectivity from Ignition, validating the redundancy / high-availability design.

# 2. Scope

•  Covers: AWS RDS Multi-AZ failover mechanism validation; Ignition platform database connection resilience during failover.

•  Environment under test: DEV (non-production), AWS region eu-west-1 (Ireland).

•  Not covered: production environment behavior (to be validated separately); application data consistency checks beyond connection validity.

# 3. Environment Under Test

| Component | Detail |
| --- | --- |
| Ignition Platform | Ignition standard 8.3.6 (b2026042713) URL: ignition-global-platform-primary-dev.apps.rosa.scale-4afdc12c.2a03.p3.ope... |
| Primary DB Instance | dev-ignition-infrastructure-db-postgres-primary |
| Secondary DB Instance | dev-ignition-infrastructure-db-postgres-secondary |
| Engine | PostgreSQL |
| Instance Class | db.t4g.large |
| Allocated Storage | 200 GB |
| AWS Region | eu-west-1 (Ireland) |
| Multi-AZ Enabled | True |
| Backup Retention Period | 14 days |

# 4. Architecture Summary

The Ignition platform is configured with two independent PostgreSQL database connections — "ignition-DB-AWS" (user: AdminUserPrimary, connects to the primary RDS endpoint) and "ignition-DB-AWS-02" (user: AdminUserSecondary, connects to the secondary RDS endpoint). Both connections use the PostgreSQL JDBC driver, with "Failover Datasource: None" and "Failover Mode: Standard" configured independently at the Ignition connection level.

In other words: Ignition relies on AWS RDS's own Multi-AZ failover mechanism (DNS-level endpoint redirection) rather than an Ignition-managed failover between the two datasources.

📷  INSERT SCREENSHOT: Ignition Edit Connection — ignition-DB-AWS, General tab (Username: AdminUserPrimary)

📷  INSERT SCREENSHOT: Ignition Edit Connection — ignition-DB-AWS, Connect URL / JDBC Driver detail

📷  INSERT SCREENSHOT: Ignition Edit Connection — ignition-DB-AWS-02, General tab (Username: AdminUserSecondary)

📷  INSERT SCREENSHOT: Ignition Edit Connection — ignition-DB-AWS-02, Connect URL detail (secondary endpoint)

# 5. Pre-Test Baseline (Before Failover)

| Parameter | Value |
| --- | --- |
| Primary Instance Status | Available |
| Primary Instance AZ | eu-west-1b |
| Secondary Instance Status | Available |
| Secondary Instance AZ | eu-west-1a |
| MultiAZ | True |
| Ignition Valid Connections | 2 / 2 |
| Ignition Errored Connections | 0 |
| Ignition Total Throughput | ~0.2 queries/sec (0.1 queries/sec per connection) |

📷  INSERT SCREENSHOT: AWS RDS Console — Databases list, pre-failover status and AZ assignment

📷  INSERT SCREENSHOT: Ignition Database Connections — pre-failover, 2/2 Valid, 0 Errored

# 6. Test Procedure

Step 1 — Confirm baseline status of both RDS instances via AWS CLI:

aws rds describe-db-instances \

        --db-instance-identifier "dev-ignition-infrastructure-db-postgres-primary" \

        --region "eu-west-1" \

        --query 'DBInstances[0].{Status:DBInstanceStatus,MultiAZ:MultiAZ,PrimaryAZ:AvailabilityZone,SecondaryAZ:SecondaryAvailabilityZone,Endpoint:Endpoint.Address}' \

        --output table

Result (baseline):

Endpoint: dev-ignition-infrastructure-db-postgres-primary.cv0sqqmowz8c.eu-west-1.rds.amazonaws.com

      MultiAZ: True   |   PrimaryAZ: eu-west-1b   |   SecondaryAZ: eu-west-1a   |   Status: available

Step 2 — Trigger a forced failover of the primary instance via AWS CLI:

aws rds reboot-db-instance \

        --db-instance-identifier "dev-ignition-infrastructure-db-postgres-primary" \

        --region "eu-west-1" \

        --force-failover

Note: --force-failover instructs RDS to fail over to the standby replica and reboot the primary, rather than performing a simple in-place reboot. This flag is only valid for Multi-AZ instances.

Step 3 — Observe the instance status transition to "Rebooting" immediately after the command executes (confirmed via both the CLI JSON response showing "DBInstanceStatus": "rebooting" and the AWS Console Databases list).

📷  INSERT SCREENSHOT: AWS RDS Console — Primary instance status showing "Reboot..." immediately after command execution, alongside CloudShell command/JSON output

Step 4 — Monitor Ignition's Database Connections page throughout the failover window for connection validity / error count.

📷  INSERT SCREENSHOT / VIDEO STILL: Ignition Database Connections during failover window

Step 5 — Once AWS reports both instances back to "Available", re-run the describe-db-instances query to confirm the post-failover AZ assignment.

aws rds describe-db-instances \

        --db-instance-identifier "dev-ignition-infrastructure-db-postgres-primary" \

        --region "eu-west-1" \

        --query 'DBInstances[0].{Status:DBInstanceStatus,MultiAZ:MultiAZ,PrimaryAZ:AvailabilityZone,SecondaryAZ:SecondaryAvailabilityZone,Endpoint:Endpoint.Address}' \

        --output table

Result (post-failover):

Endpoint: dev-ignition-infrastructure-db-postgres-primary.cv0sqqmowz8c.eu-west-1.rds.amazonaws.com

      MultiAZ: True   |   PrimaryAZ: eu-west-1a   |   SecondaryAZ: eu-west-1b   |   Status: available

📷  INSERT SCREENSHOT: AWS RDS Console — Databases list, post-failover, both Available, AZ columns showing the swap

📷  INSERT SCREENSHOT: CloudShell — post-failover describe-db-instances table output

# 7. Key Observation: Availability Zone Swap

Before the forced failover, the primary instance was running in eu-west-1b and the secondary in eu-west-1a. After the failover completed and the instance returned to "Available", the primary instance's underlying Availability Zone changed to eu-west-1a, with the secondary now in eu-west-1b — i.e. the AZ assignments swapped.

This is the definitive evidence that a genuine Multi-AZ failover occurred — the RDS endpoint DNS transparently redirected traffic to what was previously the standby instance, which has now assumed the primary role, rather than the same instance simply restarting in place.

# 8. Application-Layer Impact Assessment (Ignition Platform)

| Metric | Pre-Failover | Post-Failover |
| --- | --- | --- |
| Valid Connections | 2 / 2 | 2 / 2 |
| Errored Connections | 0 | 0 |
| Total Throughput | ~0.2 queries/sec | ~0.2 queries/sec |

Throughout the forced failover of the primary RDS instance, the Ignition platform's two configured PostgreSQL datasource connections (ignition-DB-AWS, ignition-DB-AWS-02) remained valid with zero errored connections recorded before and after the test. This demonstrates that the underlying AWS RDS Multi-AZ failover mechanism, combined with the JDBC driver's endpoint reconnection behavior, allowed the platform to tolerate the primary instance failover without requiring manual reconfiguration of the Ignition database connections.

📷  INSERT SCREENSHOT: Ignition Database Connections — post-failover, 2/2 Valid, 0 Errored, confirming continued connectivity

📷  INSERT VIDEO REFERENCE: Screen recording of failover window — see attached file [FILENAME TO BE ADDED]

# 9. Conclusion

The test successfully validates that the AWS RDS PostgreSQL database backing the Ignition DEV environment is correctly configured for Multi-AZ high availability. A forced failover of the primary instance completed within [INSERT DURATION] and resulted in a confirmed AZ swap between the primary and secondary instances, demonstrating genuine automatic failover rather than a simple reboot.

The Ignition platform's database connections remained valid and error-free throughout the test, confirming that the platform-level redundancy design meets expectations for this environment.

It is recommended that this same test be repeated / validated in the UAT and PROD environments prior to go-live sign-off.

# 10. Appendix — Raw CLI Command Log

Included for audit traceability.

Command 1 — Force failover:

aws rds reboot-db-instance \

        --db-instance-identifier "dev-ignition-infrastructure-db-postgres-primary" \

        --region "eu-west-1" \

        --force-failover

      {

        "DBInstance": {

          "DBInstanceIdentifier": "dev-ignition-infrastructure-db-postgres-primary",

          "DBInstanceClass": "db.t4g.large",

          "Engine": "postgres",

          "DBInstanceStatus": "rebooting",

          "MasterUsername": "AdminUserPrimary",

          "Endpoint": {

            "Address": "dev-ignition-infrastructure-db-postgres-primary.cv0sqqmowz8c.eu-west-1.rds.amazonaws.com",

            "Port": 5432,

            "HostedZoneId": "Z29XKXDKYMONMX"

          },

          "AllocatedStorage": 200,

          "InstanceCreateTime": "2026-02-25T12:02:20.158000+00:00",

          "PreferredBackupWindow": "02:01-02:31",

          "BackupRetentionPeriod": 14

        }

      }

Command 2 — Baseline describe (pre-failover):

aws rds describe-db-instances \

        --db-instance-identifier "dev-ignition-infrastructure-db-postgres-primary" \

        --region "eu-west-1" \

        --query 'DBInstances[0].{Status:DBInstanceStatus,MultiAZ:MultiAZ,PrimaryAZ:AvailabilityZone,SecondaryAZ:SecondaryAvailabilityZone,Endpoint:Endpoint.Address}' \

        --output table

      -------------------------------------------------------------------------------------------------------

      |                                       DescribeDBInstances                                            |

      +--------------------------------------------------------------------------------+------+------+------+

      | Endpoint                                                                        |MultiAZ|PrimAZ|SecAZ|

      +--------------------------------------------------------------------------------+------+------+------+

      | dev-ignition-infrastructure-db-postgres-primary.cv0sqqmowz8c.eu-west-1...       | True |1b    |1a    |

      | Status: available                                                               |      |      |      |

      +--------------------------------------------------------------------------------+------+------+------+

Command 3 — Post-failover verification:

aws rds describe-db-instances --db-instance-identifier "dev-ignition-infrastructure-db-postgres-primary" --region "eu-west-1" \

        --query 'DBInstances[0].{Status:DBInstanceStatus,MultiAZ:MultiAZ,PrimaryAZ:AvailabilityZone,SecondaryAZ:SecondaryAvailabilityZone,Endpoint:Endpoint.Address}' --output table

      -------------------------------------------------------------------------------------------------------

      |                                       DescribeDBInstances                                            |

      +--------------------------------------------------------------------------------+------+------+------+

      | Endpoint                                                                        |MultiAZ|PrimAZ|SecAZ|

      +--------------------------------------------------------------------------------+------+------+------+

      | dev-ignition-infrastructure-db-postgres-primary.cv0sqqmowz8c.eu-west-1...       | True |1a    |1b    |

      | Status: available                                                               |      |      |      |

      +--------------------------------------------------------------------------------+------+------+------+